GOVERNMENT OF GUJARAT

FOREST DEPARTMENT

BGDSS

Beat GIS Decision Support System

A Satellite Data-Driven Model for Forest Restoration Site Selection

Pilot: Raygadh Range, Sabarkantha Forest Division

Prepared for PMO / Leadership Review

Built on Google Earth Engine using GEDI, Sentinel-1/2, Copernicus DEM, MODIS, ISRIC SoilGrids and Hansen Global Forest Change Fourteen scored criteria. One reproducible, auditable ranking. Zero field survey cost to generate the first pass.

# 1.  Executive Summary

BGDSS is a Google Earth Engine-based decision support system that ranks every hectare of a forest beat for restoration priority — automatically, reproducibly, and using only publicly available satellite data. This briefing covers Module 2, the Plantation / Eco-Restoration Suitability Model, piloted on a beat in Raygadh Range, Sabarkantha Forest Division.

Instead of nominating restoration sites by field familiarity alone, the model scores every eligible hectare on 14 independent criteria — terrain, soil, hydrology, climate, biomass, invasive species pressure, fire history, and human accessibility — each weighted by explicit, stated logic, and combined into a single, rankable Priority Score. The output is a ranked list of blocks, ready to match against whatever hectare target a funding cycle actually allows.

## Why this matters for decision-makers

- Removes guesswork from site selection: every recommended hectare is traceable to a specific, named, citable reason.

- Uses the same class of satellite instruments used in international climate and conservation science — not assumptions or anecdote.

- Runs the entire beat in minutes, at zero incremental field-survey cost for the first pass.

- Scales directly: the same script re-run on a different beat boundary produces the same rigor, Range-wide or Division-wide.

- Every score, weight, and dataset is documented and auditable — this briefing is that documentation.

STATUS: This is the pilot stage. Two of the fourteen criteria are running on neutral placeholders pending two data layers (village/road locations, wildlife corridor boundary) that the Range can supply at any time — see Section 7.

# 2.  The Approach

## The problem with ad hoc site selection

Restoration funding is finite, and site selection has traditionally relied on field officer familiarity with the terrain. That knowledge is valuable but not scalable, not easily audited by a funding authority, and not able to weigh a dozen factors simultaneously and consistently across every hectare of a beat.

## The BGDSS approach

Every pixel of the beat is tested against a set of hard exclusion rules first (already-forested land, water, cropland, built-up areas, unworkable slopes, bare rock) to isolate genuinely eligible land. Every eligible hectare is then scored on 14 weighted criteria, each stretched to reflect the real range of conditions found in this specific beat — not a generic assumption imported from elsewhere. The result is one Priority Score per hectare, and a ranked, exportable list of blocks ready for field verification and funding submission.

# 3.  Technology Foundation

The model runs entirely on Google Earth Engine, drawing on the same publicly available satellite missions used in international forestry and climate science:

| Data Source | What It Provides |
| --- | --- |
| GEDI (NASA, spaceborne lidar aboard the ISS) | Direct measurement of forest biomass and canopy structure — not a greenness proxy |
| Sentinel-1 & Sentinel-2 (ESA Copernicus) | High-resolution optical and radar imagery — vegetation health, land cover, seasonal change |
| Copernicus DEM GLO-30 | 30 m global elevation model — slope, aspect, terrain position |
| MERIT Hydro | Drainage network and terrain-moisture positioning |
| MODIS (NASA Aqua/Terra) + Landsat 8 | Land surface temperature and 20+ years of fire history |
| ISRIC SoilGrids v2 | Global soil property maps — water-holding capacity, organic carbon, texture |
| Hansen Global Forest Change (University of Maryland) | Annually updated global forest cover and loss |
| ESA WorldCover / Google Dynamic World | Detailed, current land-cover classification |
| CHIRPS (UCSB Climate Hazards Group) | 40+ years of daily rainfall data |

All of this is free, public, continuously updated, and independently verifiable — anyone with Earth Engine access can reproduce this exact analysis.

# 4.  The 14-Criterion Model

Every criterion below is built from a named, public dataset, scored on a 0–100% scale calibrated to this beat’s own real conditions, and combined by weight into one Priority Score. The weights were set deliberately, not guessed — heavier weight on the factors most responsible for restoration success or failure in this landscape.

| Criterion | Weight | Data Source | Why It Matters |
| --- | --- | --- | --- |
| Soil Depth | 15% | Composite: terrain shape, drainage position, soil coarseness, satellite bare-rock detection | Shallow soil is the single biggest driver of restoration failure. No global depth-to-bedrock map is reliable enough alone, so 5 independent physical signals are combined. |
| Soil Texture (Water-Holding Capacity) | 7% | ISRIC SoilGrids (field capacity minus wilting point) | A direct physical measure of how much water the soil can hold and deliver to a sapling through the dry season. |
| Soil Organic Carbon | 6% | ISRIC SoilGrids | Higher organic carbon means better site fertility and higher restoration potential. |
| Distance to Existing Forest | 10% | Hansen Global Forest Change + WorldCover | Land near real forest has a natural seed source; regeneration and survival are measurably better close to it. |
| Fragmentation Index | 10% | Local forest density (Hansen) | Prioritizes consolidating and expanding existing forest patches over creating new, isolated islands. |
| Hydrological Connectivity | 8% | Water bodies, drainage network, valley-position modeling (MERIT Hydro) | Land near water or in moisture-retaining valley positions gives a better ecosystem-service return and survival rate. |
| Wildlife Corridor Proximity | 8% | Forest Department corridor boundary (pending) | Restoring the buffer around a known wildlife corridor improves its effective width and habitat quality. |
| Workability (Slope & Rainfall) | 7% | Copernicus DEM + CHIRPS rainfall | Gentler slope and adequate rainfall mean lower-cost, higher-success establishment. |
| Invasive Species Pressure | 7% | Sentinel-2 seasonal greenness pattern | Detects the signature of Lantana camara (the dominant local invasive) so infested sites are costed and treated correctly, not just planted over. |
| Carbon-Gain Potential | 6% | GEDI biomass vs. this beat’s own best forest as the benchmark | The larger the gap between a site’s current carbon stock and healthy nearby forest, the greater the climate benefit of restoring it. |
| Climate Exposure | 5% | MODIS/Landsat heat + rainfall variability + slope-aspect sun exposure | Hotter, more drought-prone, more sun-exposed sites are a riskier establishment bet and are deprioritized accordingly. |
| Soil Erosion Risk | 5% | RUSLE model (Indian rainfall-erosivity formula, soil, slope, land cover) | Higher erosion risk raises priority, since restoration here carries a soil-and-water-conservation co-benefit. |
| Fire Frequency History | 3% | MODIS 10-year burned-area record | Frequently burned land is a higher-risk investment and is flagged for fireline budget rather than planted blind. |
| Biotic Pressure & Access | 3% | Village/road proximity (pending) | Distance from settlement is treated as a grazing-pressure indicator (this is a restoration programme, not community-managed forestry); road access controls real establishment cost. |

Weights sum to exactly 100% — the underlying script verifies this automatically before it will run.

# 5.  How the Priority Score Is Calculated

## Step 1 — Eligibility

Land already well-stocked with forest, water bodies, cropland, built-up areas, bare rock, and slopes beyond practical planting limits are excluded entirely before any scoring happens. Only genuinely available land is scored.

## Step 2 — Local calibration

Each of the 14 criteria is stretched against the real 2nd–98th percentile range found within this beat’s own eligible land — not a fixed, generic assumption. This is what makes the map reflect actual local conditions rather than an imported guess.

## Step 3 — Weighted combination

Every criterion is combined into one Priority Score using the weights in Section 4. Higher score = higher restoration priority, considering all 14 factors simultaneously.

## Step 4 — Ranking

Every ~1 hectare block in the beat is ranked highest-to-lowest. The top-ranked blocks, up to whatever hectare target a funding cycle allows, are the ones proposed — so the recommendation always matches the real budget, not a hypothetical one.

# 6.  Scientific Rigor & Transparency

A model built for a government funding submission has to survive scrutiny, not just look convincing. Three safeguards are built directly into BGDSS:

## Weights are not guessed

The weighting was derived using the Analytic Hierarchy Process (AHP) — the standard, peer-reviewed method (Saaty, 1980) for converting expert judgment into defensible numeric weights through structured pairwise comparison. A companion interactive tool lets the Range/DFO team run this exercise themselves and export a fully auditable weight set, including a mathematical consistency check.

## Redundancy is measured, not assumed

Several physical factors (slope, rainfall) genuinely influence more than one criterion. Rather than ignore this, the model runs a correlation diagnostic across all 14 criteria on every run and flags any pair that may be double-counting the same underlying signal.

## Robustness is tested, not claimed

A built-in sensitivity analysis re-runs the ranking with each individual weight shifted ±20% and reports how many top-ranked sites remain top-ranked regardless. Sites that survive every perturbation are the genuinely robust recommendations; sites that move are flagged as weight-sensitive.

STATUS: Every criterion raster, the full ranked block list, the correlation matrix, and the sensitivity results are exported automatically on every run — nothing in this model is a black box.

# 7.  Current Status & What’s Pending

| Item | Status | What’s Needed |
| --- | --- | --- |
| Core 12 of 14 criteria | Live, running on real satellite data | None |
| Wildlife Corridor Proximity (8% weight) | Running on neutral placeholder | Forest Department corridor boundary layer |
| Biotic Pressure & Access (3% weight) | Running on partial/coarse placeholder | Village point + road line layers |
| Soil Depth field calibration | Composite satellite proxy (15% weight) | 30–50 GPS-located soil-pit depths would upgrade this to a locally-trained model — the single highest-value field investment available |
| Scale | Piloted on one beat | Ready to extend Range-wide or Division-wide with no methodology change |

# 8.  Recommendation to PMO

- Endorse the BGDSS methodology as the standard site-selection process for CAMPA/restoration funding submissions from this Division.

- Direct the Range to supply the two pending data layers (village/road locations, wildlife corridor boundary) to close the remaining accuracy gap at no additional cost.

- Approve a modest field-verification budget (30–50 soil-pit GPS points) to upgrade the model’s largest-weighted criterion from a satellite proxy to a field-calibrated measurement.

- Authorize scaling the same model, unchanged, to Range- or Division-wide site prioritization for the next funding cycle.

BGDSS turns site selection from a matter of individual field judgment into a documented, reproducible, and auditable process — exactly the standard a PMO-level funding submission needs to withstand scrutiny.