GOVERNMENT OF GUJARAT

FOREST DEPARTMENT

CAMPA SITE SELECTION JUSTIFICATION

Detailed Project Report (Draft)

Beat GIS Decision Support System (BGDSS) - Pilot Project

Sabarkantha Forest Division

Raygadh Range

Prepared under the CAMPA (Compensatory Afforestation Fund) framework

for submission to the State CAMPA / Annual Plan of Operations (APO)

DRAFT v0.1 - NOT FOR FINAL SUBMISSION This document is a methodology-complete FRAMEWORK, generated from the BGDSS GIS site-prioritization pipeline. Site-level areas, costs, and carbon figures marked "ASSUMPTION - VERIFY" must be populated from an actual Module 2 (CAMPA Site Prioritization) run and the Range’s confirmed CAMPA APO allocation before this leaves draft status. See Annexure C for the complete assumptions register.

# 1.  Executive Summary

This report justifies the selection of treatment sites within the pilot Beat (Raygadh Range, Sabarkantha Forest Division, ~11,377 ha) for funding under the Compensatory Afforestation Fund Management and Planning Authority (CAMPA) framework, using a reproducible, GIS-based site-prioritization methodology built on the Beat GIS Decision Support System (BGDSS).

Rather than nominating sites by field familiarity alone, every hectare of the beat was scored on four criteria - degradation severity, soil erosion risk, carbon-gain potential, and site workability - using publicly verifiable satellite datasets and India-specific methodologies (Forest Survey of India / ISFR carbon convention, CSWCRTI rainfall-erosivity, NRSC Soil Erosion Atlas land-cover factors). The highest-ranked hectares, up to the Range’s funded target area, are proposed for treatment. Every criterion, weight, and dataset used is stated explicitly so the ranking can be audited or re-run with better local data at any time.

## Highlights

- Study area: ~11,377 ha, Champion & Seth (1968) Group 5B - Northern Dry Deciduous Forest.

- Three fundable treatment types identified per hectare: New Plantation, Assisted Natural Regeneration (ANR), and Soil & Water Conservation works.

- Site selection is GIS-ranked and reproducible: the same script can be re-run for any beat in the Division by changing one line of configuration.

- Target treatment area, cost, and carbon-gain figures below are placeholders pending an actual run of the Site Prioritization module and the Range’s confirmed APO figure (see Section 5 and Annexure C).

ASSUMPTION - VERIFY: All hectare counts, cost totals, and carbon-gain figures in this draft are structurally correct but numerically illustrative. Run BGDSS_Module2_CAMPASitePrioritization.js against the confirmed target area before using this document for an actual funding submission.

# 2.  Legal & Policy Basis

This proposal is framed to align with the following statutory and policy instruments governing compensatory afforestation and forest-restoration funding in India:

- Compensatory Afforestation Fund Act, 2016 (CAF Act 2016) - establishes the National and State Compensatory Afforestation Fund Management and Planning Authorities (CAMPA) and the legal basis for fund collection and utilization.

- Compensatory Afforestation Fund Rules, 2018 (CAF Rules 2018) - operationalizes the Act, defining fund-utilization heads including Compensatory Afforestation, Additional Compensatory Afforestation (which covers assisted natural regeneration, soil and moisture conservation works, forest protection infrastructure, and silvicultural operations), Net Present Value (NPV) utilization, and Catchment Area Treatment.

- The Annual Plan of Operations (APO) mechanism, through which the State CAMPA (Gujarat) approves site-specific schemes and releases funds against them.

This document is prepared as a site-selection justification to accompany a scheme submission under the Range’s APO. It does not itself constitute an approved scheme; the Range/Division must route it through the standard State CAMPA APO approval process.

ASSUMPTION - VERIFY: Verify the current Gujarat State CAMPA APO guidelines, the specific fund head this scheme will be routed under, and the applicable per-hectare unit cost schedule with the Division’s CAMPA cell before submission - these are administratively specific and subject to periodic revision.

# 3.  Study Area

| Attribute | Value |
| --- | --- |
| State | Gujarat |
| District | Sabarkantha |
| Division | Sabarkantha Forest Division |
| Range | Raygadh Range |
| Unit analyzed | Beat (asset: projects/raygadh-range/assets/BEAT) |
| Total area | ~11,377 ha (computed by BGDSS from the beat boundary) |
| Forest type | Champion & Seth (1968) Group 5B - Northern Dry Deciduous Forest (dry teak-bearing / mixed, with dry deciduous scrub in degraded tracts) |
| Analysis year | 2025 |

Note: the analyzed unit spans an area larger than a typical Forest Beat (commonly 500-3,000 ha in Indian forest administration); confirm whether the supplied boundary represents a single beat, a beat group, or a Range subdivision before finalizing the scheme name and jurisdiction in the APO submission.

# 4.  Methodology: GIS-Based Site Prioritization

## 4.1  Data Sources

| Dataset | Resolution | Source | Used For |
| --- | --- | --- | --- |
| JAXA ALOS AW3D30 (DSM) | 30 m | JAXA / Earth Engine public catalog | Elevation, slope (workability, RUSLE-LS) |
| ESA WorldCover v200 | 10 m | ESA / Earth Engine public catalog | Land-cover mask (excludes cropland/built-up/water) |
| Hansen Global Forest Change | 30 m | UMD / Earth Engine public catalog | Canopy density -> degradation / treatment eligibility |
| CHIRPS Daily | ~5.5 km (resampled) | UCSB CHG / Earth Engine public catalog | Annual rainfall (workability, RUSLE-R) |
| ESA CCI Biomass (or Sentinel-2 NDVI proxy) | ~100 m / 10 m | ESA CCI / Copernicus | Current above-ground biomass (carbon-gain potential) |

## 4.2  Treatment Eligibility

Every 10 m pixel is classified by canopy density (Hansen treecover2000), gated by an ESA WorldCover mask that excludes cropland, built-up, and water classes regardless of canopy reading (CAMPA does not fund treatment on these):

| Canopy Density | Classification | Rationale |
| --- | --- | --- |
| < 10% | New Plantation candidate | Effectively unstocked; standard threshold for block plantation over regeneration |
| 10% - 40% | Assisted Natural Regeneration (ANR) candidate | Existing root-stock/coppice potential - the standard Indian working-plan threshold favouring regeneration over planting |
| > 40% | Not prioritized | Reasonably stocked already; lower marginal benefit from CAMPA treatment |

## 4.3  Priority Scoring Criteria & Weights

Eligible pixels are scored 0-1 on four criteria and combined by a weighted overlay. Weights are a defensible starting point set by the BGDSS pilot team and should be reviewed by the Range/DFO with local silvicultural judgment before the ranking is treated as final.

| Criterion | Weight | Definition |
| --- | --- | --- |
| Degradation severity | 35% | Lower canopy density scores higher (more room for gain) |
| Soil erosion risk (RUSLE) | 30% | Higher modelled soil loss (t/ha/yr) scores higher (Soil & Water Conservation co-benefit) |
| Carbon-gain potential | 20% | Deficit between current biomass and a reference dry-deciduous AGB benchmark |
| Site workability | 15% | Gentler slope and adequate rainfall score higher (establishment feasibility) |

## 4.4  Block-Level Aggregation & Ranking

Pixel-level scores are aggregated into ~9 ha planning blocks (a practical fencing/protection unit size, not a single pixel) using a regular grid over the beat boundary. Each block is assigned its mean priority score, dominant treatment type, mean soil loss, and mean carbon deficit. Blocks are then ranked highest-priority first, and selected by cumulative area until the Range’s funded target area is reached - so the proposal contains exactly as many hectares as are actually fundable this cycle, drawn from the best-ranked ground first.

## 4.5  Reproducibility

The full scoring and ranking pipeline is implemented in a single, auditable Google Earth Engine script (BGDSS_Module2_CAMPASitePrioritization.js) using only publicly available satellite datasets. Any reviewer with Earth Engine access can re-run it against the same beat boundary and reproduce this ranking exactly, or re-run it against a different beat by changing one configuration line - the same pipeline scales to Range- or Division-wide site selection.

# 5.  Site Selection Results

ASSUMPTION - VERIFY: The table below is a TEMPLATE. Run BGDSS_Module2_CAMPASitePrioritization.js with CONFIG.targetTreatmentAreaHa set to the Range’s confirmed APO figure, then paste the printed summary and the top rows of BGDSS_CAMPA_RankedBlocks.csv here.

## 5.1  Summary

| Metric | Value |
| --- | --- |
| Total eligible area (beat-wide) | [from Console: "X ha total eligible"] |
| Target treatment area (this cycle) | [CONFIG.targetTreatmentAreaHa, ha] |
| Selected area (actual, best-ranked) | [from Console: selected ha] |
| New Plantation area | [sum of selected blocks where Recommended Treatment = New Plantation] |
| ANR area | [sum of selected blocks where Recommended Treatment = ANR] |
| Estimated cost (placeholder rates) | [from Console, flag as ASSUMPTION] |
| Estimated carbon gain (10-yr horizon) | [from Console, tonnes C / tonnes CO2e] |

## 5.2  Top-Ranked Blocks (populate from BGDSS_CAMPA_RankedBlocks.csv)

| Rank | Block ID | Area (ha) | Priority Score | Treatment | Soil Loss (t/ha/yr) |
| --- | --- | --- | --- | --- | --- |
| 1 | [ ] | [ ] | [ ] | [ ] | [ ] |
| 2 | [ ] | [ ] | [ ] | [ ] | [ ] |
| 3 | [ ] | [ ] | [ ] | [ ] | [ ] |
| 4 | [ ] | [ ] | [ ] | [ ] | [ ] |
| 5 | [ ] | [ ] | [ ] | [ ] | [ ] |

Supporting maps (attach as figures): CAMPA Priority Score raster, Recommended Treatment Type raster, and the Planning Blocks boundary vector - all exported by Module 2 in EPSG:32643 for direct ArcGIS overlay.

# 6.  Proposed Treatment Package

## 6.1  New Plantation

Indicative species palette for Northern Dry Deciduous conditions (Champion & Seth Group 5B), pending confirmation against the Range’s current Working Plan species prescription:

| Species | Rainfall Range (mm) | Notes |
| --- | --- | --- |
| Tectona grandis (Teak) | 900 - 2,500 | Primary Working Plan species for this forest type where soil depth permits |
| Azadirachta indica (Neem) | 400 - 1,200 | Hardy, drought-tolerant; suitable for drier/shallower sites |
| Acacia nilotica (Babul) | 300 - 1,200 | Suitable for degraded/saline-prone patches |
| Dendrocalamus strictus (Bamboo) | 750 - 2,000 | Fast establishment; soil-binding value on erosion-prone blocks |

Standard practice: pit size 45x45x45 cm, spacing per species/Working Plan norm (typically 2x2 m to 3x3 m for block plantation), pre-monsoon pitting, monsoon-onset planting, and fencing/trenching for protection from grazing.

## 6.2  Assisted Natural Regeneration (ANR)

For blocks with existing root-stock/coppice potential (10-40% canopy): soil working around existing stumps/seedlings, climber cutting, protection from grazing and fire, and gap-filling planting only where natural regeneration density is insufficient. ANR is markedly lower-cost than new plantation and is the CAMPA-preferred treatment wherever ecologically justified.

## 6.3  Soil & Water Conservation Works

For blocks with high RUSLE soil-loss scores: contour trenching, loose boulder/brushwood check structures on drainage lines, and staggered contour trenches on steeper micro-slopes, sized to the block’s modelled soil-loss rate and slope class.

# 7.  Carbon & Ecosystem Service Outcomes

Carbon-gain potential is estimated as the deficit between each block’s current above-ground biomass (AGB) and a reference AGB benchmark for a well-stocked stand of this forest type, converted to carbon stock using the Forest Survey of India / India State of Forest Report (ISFR) convention.

## Method

- AGB deficit (t/ha) = Reference AGB benchmark - Current AGB (from ESA CCI Biomass, or an NDVI-based proxy where unavailable)

- Below-ground biomass = AGB deficit x 0.28 (IPCC 2006 Guidelines for National GHG Inventories, Vol. 4, Table 4.4 - tropical/subtropical dry forest root-shoot ratio, AGB > ~20 t/ha)

- Carbon stock gain = (AGB deficit + below-ground biomass) x 0.5 (carbon fraction; FSI/ISFR convention - set to 0.47 to match the IPCC 2006 Tier-1 international default instead)

- CO2-equivalent = Carbon stock gain x 3.6663 (44/12 molecular weight ratio)

ASSUMPTION - VERIFY: The reference AGB benchmark (currently a planning-stage placeholder) should be replaced with the Gujarat-specific figure from the current ISFR or the Division’s Working Plan volume tables. As a cross-check on any allometric biomass estimate derived from field DBH/height data, use the Chave et al. (2014) pantropical model as the primary estimator, with Chave et al. (2005) Model I as a dry-forest sensitivity check.

The carbon-gain figure quoted in Section 5 is a standing-stock gain over the stated restoration horizon, not an annual sequestration rate - report both consistently in any funding submission to avoid a common reviewer misread.

# 8.  Cost Estimate

ASSUMPTION - VERIFY: The rates below are commonly cited Indian CAMPA/afforestation planning figures for a 3-year-maintenance plantation package - they are NOT a Gujarat-specific rate. Replace with the current Gujarat Forest Department / State CAMPA APO Schedule of Rates before this figure is quoted in a funding submission.

| Treatment | Indicative Rate (Rs/ha) | Basis |
| --- | --- | --- |
| New Plantation (incl. 3-yr maintenance, fencing) | 65,000 (placeholder) | Commonly cited CAMPA block-plantation planning range |
| Assisted Natural Regeneration (ANR) | 30,000 (placeholder) | Lower cost - no nursery stock, lighter site preparation |
| Soil & Water Conservation works | 45,000 (placeholder) | Contour trenching / check-structure planning range |

Total scheme cost = sum over selected blocks of (block area x rate for that block’s recommended treatment) - computed automatically by Module 2 and printed to Console once run. Tie the final figure to the current Gujarat SoR / CPWD DSR analysis-of-rates build-up for the APO submission rather than the placeholder above.

# 9.  Implementation Phasing

| Year | Activity |
| --- | --- |
| Year 1 (pre-monsoon) | Site demarcation, fencing/trenching, pitting for New Plantation blocks; ANR site preparation |
| Year 1 (monsoon) | Planting; ANR gap-filling and climber cutting |
| Year 2 | Casualty replacement, weeding, soil-moisture conservation works execution |
| Year 3 | Continued maintenance, protection, and first survival census |
| Year 4+ | Handover to regular protection regime; satellite-based MRV monitoring continues (Section 11) |

# 10.  Risk Assessment & Mitigation

| Risk | Likelihood | Mitigation |
| --- | --- | --- |
| Fire damage to new plantation/ANR blocks | Moderate-High (Sabarkantha is Champion & Seth Group 5B, a fire-prone dry-deciduous type) | Cross-reference selected blocks against the BGDSS Fire Risk Zonation module (Module 1, Jaiswal et al. 2002 AHP); prioritize fire-line clearance and pre-monsoon patrol around any selected block falling in a High/Very High fire-risk zone |
| Grazing pressure / browsing damage | Moderate | Fencing/trenching per standard practice; community engagement where village proximity is high |
| Drought-related establishment mortality | Moderate (semi-arid rainfall regime) | Species selection weighted to site rainfall (Section 6.1); irrigation/watering in first dry season where feasible |
| Encroachment / land-use conflict | Low-Moderate | Confirm land-record status (notified forest vs. revenue land handed over for CA) before finalizing block boundaries |

# 11.  Monitoring, Reporting & Verification (MRV)

Because the site-selection pipeline is built entirely on satellite data, the same BGDSS scripts can be re-run annually against the treated blocks to track progress without additional field survey cost:

- Canopy/NDVI trend on New Plantation and ANR blocks, year-over-year, as a survival/establishment proxy

- Hansen/Sentinel-2-based canopy-gain tracking against the pre-treatment baseline captured in this report

- Re-run of the Soil Erosion (RUSLE) layer post-treatment to quantify erosion-reduction outcomes on Soil & Water Conservation blocks

- Re-run of the Carbon/Biomass layer at the end of the restoration horizon to report actual vs. projected carbon gain

# 12.  Maintenance Strategy

Standard CAMPA 3-year maintenance regime: casualty replacement in Year 2, continued weeding and fire-line maintenance through Year 3, and beat-guard patrol integrated into the Range’s regular protection duties from Year 4 onward. Maintenance intensity should be sized to the Range’s actual field-staff strength - do not propose a maintenance schedule the beat cannot realistically staff.

# 13.  Innovation & Value Addition

- GIS-ranked, auditable site selection replaces ad hoc nomination - every hectare’s inclusion is traceable to a stated criterion and weight.

- The same pipeline scales directly to Range- or Division-wide site selection by changing one configuration line, with no re-engineering.

- Built entirely on free, publicly available satellite data and open, documented Indian forestry methodology - reproducible by any reviewer with Earth Engine access.

- Establishes a pre-treatment baseline (canopy, biomass, erosion) that makes post-treatment MRV (Section 11) directly comparable, strengthening future funding submissions with demonstrated outcomes rather than projections alone.

# 14.  Annexures

## Annexure A: Full Ranked Block List

See BGDSS_CAMPA_RankedBlocks.csv (exported by Module 2) for every block in the beat, its priority score, recommended treatment, and selection status.

## Annexure B: GIS Reproducibility Statement

Site selection was performed using BGDSS_Module2_CAMPASitePrioritization.js on the Google Earth Engine platform, against publicly available datasets listed in Section 4.1. The script, all weights, and all thresholds are stated in full in this report and in the script’s configuration block; no proprietary or undisclosed data or methodology was used.

## Annexure C: Assumptions Register

Every figure in this draft that requires local verification before submission:

| # | Assumption | Replace With |
| --- | --- | --- |
| 1 | Target treatment area: 200 ha (illustrative) | Range's confirmed CAMPA APO allocation for this cycle |
| 2 | Cost rates: Rs 65,000 / 30,000 / 45,000 per ha (New Plantation / ANR / S&WC) | Current Gujarat Forest Department / State CAMPA APO Schedule of Rates |
| 3 | Reference AGB benchmark: 120 t/ha | Gujarat-specific figure from the current ISFR or the Division's Working Plan volume tables |
| 4 | Soil erodibility (K-factor): flat constant 0.28 (loam average) | OpenLandMap-derived K-factor for higher precision (see full BGDSS Soil Engine) |
| 5 | Priority-score weights (35/30/20/15%) | Range/DFO team's own judgment, ideally via a documented AHP pairwise comparison |
| 6 | Species palette (Section 6.1) | Range's current Working Plan prescribed species list |
| 7 | Land-record status of selected blocks | Confirm notified-forest vs. handed-over-non-forest status per block before finalizing |